TensorRT GPU Pipeline

Face Recognition — GPU-Accelerated with NVIDIA Jetson

Complete Pipeline Documentation | Setup Guide | Performance Analysis

# 1. System Overview

This implementation uses NVIDIA TensorRT to accelerate both face detection (RetinaFace) and face recognition (MobileFaceNet) on NVIDIA Jetson GPU hardware. TensorRT optimizes ONNX models for maximum GPU performance through layer fusion, kernel auto-tuning, and precision calibration.

## Key Technologies

- TensorRT 10.3.0 — NVIDIA's high-performance deep learning inference engine

- CUDA 12.6 — Parallel computing platform for GPU execution

- PyCUDA — Python interface for CUDA memory management

- Unified Memory — Jetson's shared CPU/GPU memory architecture

- Async API — Background threading for non-blocking attendance logging

- Adaptive Threshold — Optional brightness-based matching (disabled by default)

## System Requirements

| Component | Minimum | Recommended |
| --- | --- | --- |
| Hardware | Jetson Orin Nano 8GB | Jetson Orin Nano 8GB |
| JetPack | 6.0+ | 6.2.1 |
| CUDA | 12.0+ | 12.6 |
| TensorRT | 10.0+ | 10.3.0 |
| cuDNN | 8.6+ | 8.9.7 |
| Python | 3.8+ | 3.10 |
| PyCUDA | 2022.1+ | 2024.1 |
| OpenCV | 4.5+ | 4.8 (CUDA-enabled) |

Note: This code is specifically designed for NVIDIA Jetson platforms with TensorRT support. It will NOT run on standard x86 CPUs or non-NVIDIA GPUs.

# 2. Complete Pipeline Structure

The pipeline processes each video frame through 15 distinct stages, utilizing both CPU and GPU resources optimally.

| # | Stage | Input | Output | Hardware |
| --- | --- | --- | --- | --- |
| 1 | Startup | Config files | Models in GPU | GPU VRAM |
| 2 | Video Capture | Camera/File | BGR frame | CPU |
| 3 | Preprocess | Frame | 640×640 blob | CPU |
| 4 | GPU Upload | Blob | GPU memory | PCIe/Unified |
| 5 | Detection | GPU memory | Boxes+scores | GPU (TRT) |
| 6 | GPU Download | GPU memory | CPU arrays | PCIe/Unified |
| 7 | NMS | Boxes+scores | Final boxes | CPU |
| 8 | Face Extract | Frame+boxes | Face crops | CPU |
| 9 | Threshold Calc | Face crop | Threshold val | CPU |
| 10 | GPU Upload | Face crop | GPU memory | PCIe/Unified |
| 11 | Recognition | GPU memory | 512D embedding | GPU (TRT) |
| 12 | GPU Download | GPU memory | CPU array | PCIe/Unified |
| 13 | Matching | Embedding+DB | ID+score | CPU |
| 14 | API Queue | ID+score | Queue entry | CPU |
| 15 | Display | Annotated | Window | CPU |

## Pipeline Flow Diagram

[ VIDEO INPUT ]

↓

[ CPU: Preprocess 640×640 ]

↓

[ GPU Upload via PyCUDA ] ← Memory transfer

↓

[ GPU: RetinaFace TensorRT ] ← 3-5ms

↓

[ GPU Download via PyCUDA ] ← Memory transfer

↓

[ CPU: NMS + Extract Faces ]

↓

[ GPU Upload Face Crop ] ← Memory transfer

↓

[ GPU: MobileFaceNet TensorRT ] ← 1-2ms

↓

[ GPU Download Embedding ] ← Memory transfer

↓

[ CPU: Cosine Similarity Match ]

↓

[ ASYNC API: Queue Request ]

↓

[ DISPLAY: Annotated Frame ]

# 3. Stage-by-Stage Breakdown

## Stage 1: Startup — Load TensorRT Engines

rf_trt = TRTInferencer(RETINAFACE_ENGINE, ...)

mf_trt = TRTInferencer(MOBILEFACENET_ENGINE, ...)

### What Happens:

- Load serialized TensorRT engine files (.engine format)

- Allocate GPU memory buffers for inputs/outputs

- Create CUDA execution context

- Initialize CUDA stream for async operations

### Memory Layout:

RetinaFace Input:  (1, 3, 640, 640) FP32 → 4.9 MB

RetinaFace Output: Boxes (16800,4) + Scores (16800,2) → 0.3 MB

MobileFaceNet Input:  (1, 3, 112, 112) FP32 → 150 KB

MobileFaceNet Output: (512,) FP32 → 2 KB

### Why TensorRT vs ONNX Runtime:

- TensorRT: GPU-optimized, layer fusion, 4-6x faster

- ONNX Runtime: CPU execution, no optimization, baseline speed

## Stage 2: Video Capture

cap = cv2.VideoCapture(VIDEO_SOURCE)

ret, frame = cap.read()  # Each frame

- Hardware: CPU

- Reads BGR frame from camera or video file

- Typical resolution: 1920×1080 or 640×480

- No GPU involved at this stage

## Stage 3: Preprocessing (CPU)

img = cv2.resize(frame, (640,640)).astype(np.float32)

img -= np.array([104.0, 117.0, 123.0])  # ImageNet mean

blob = img.transpose(2,0,1)[None,...]   # NCHW format

- Hardware: CPU

- Resize to 640×640 (RetinaFace input size)

- Subtract mean values for normalization

- Transpose HWC → CHW (PyTorch/ONNX format)

- Add batch dimension: (H,W,C) → (1,C,H,W)

## Stage 4: GPU Upload (PyCUDA)

np.copyto(inp['h'], arr.ravel())         # CPU to pinned

cuda.memcpy_htod_async(inp['d'], inp['h']) # Pinned to GPU

### Memory Transfer Path:

- CPU memory → Pinned (page-locked) memory → GPU VRAM

- Async transfer: CPU continues while GPU copies

- Transfer time: ~0.5-1 ms for 640×640×3 blob

### Jetson Optimization:

- Unified memory architecture (shared CPU/GPU RAM)

- Zero-copy possible in some cases

- Faster than PCIe transfer on desktop GPUs

## Stage 5: Face Detection (TensorRT GPU)

self.context.execute_async_v3(self.stream.handle)

### Hardware: GPU (512 CUDA cores on Orin Nano)

- TensorRT executes optimized RetinaFace model

- Layer fusion: Conv+BN+ReLU combined into single kernel

- FP32 precision (can use FP16 for 2x speedup)

- Inference time: 3-5 ms (vs 25-35 ms on CPU)

### Output:

- output0: Bounding box offsets [16800, 4]

- softmax: Class probabilities [16800, 2] (background vs face)

## Stage 6: GPU Download (PyCUDA)

cuda.memcpy_dtoh_async(bufs[n]['h'], bufs[n]['d'])

- Transfer: GPU VRAM → Pinned memory → CPU memory

- Async operation: overlaps with next frame preprocessing

- Transfer time: ~0.3-0.5 ms for detection outputs

## Stage 7: NMS (CPU)

boxes = decode_boxes(loc[0], anchors)

keep = nms(boxes, scores, NMS_THRESH)

### Hardware: CPU

- Decode anchor-based predictions to absolute coordinates

- Apply Non-Maximum Suppression to remove duplicates

- IoU threshold: 0.4 (overlap removal)

- Confidence filter: 0.7 (minimum score)

- Processing time: 3-5 ms

## Stage 8: Face Extraction (CPU)

face = frame[y1:y2, x1:x2]

- Hardware: CPU

- Crop face regions from original frame

- Clamp coordinates to frame boundaries

- Validate non-empty regions

- Processing time: <1 ms per face

## Stage 9: Adaptive Threshold Calculation (CPU)

brightness = float(np.mean(gray))

threshold = DARK if brightness < 80 else BRIGHT

- Hardware: CPU

- Convert face crop to grayscale

- Calculate mean pixel intensity (0-255)

- Select threshold based on brightness

- Processing time: <0.1 ms

## Stage 10: GPU Upload Face Crop

fr = cv2.resize(face, (112,112)).astype(np.float32)

fr = (fr - 127.5) / 127.5  # Normalize to [-1,1]

cuda.memcpy_htod_async(...)  # Upload to GPU

- Resize face to 112×112 (MobileFaceNet input)

- Normalize pixels: [0,255] → [-1,1]

- Transpose HWC → CHW

- Upload to GPU: ~0.1 ms (small 112×112 image)

## Stage 11: Face Recognition (TensorRT GPU)

self.context.execute_async_v3(self.stream.handle)

### Hardware: GPU

- TensorRT executes optimized MobileFaceNet

- Generates 512-dimensional embedding vector

- L2-normalized output (unit length)

- Inference time: 1-2 ms (vs 5-8 ms on CPU)

### Output:

- 516: Embedding vector [512] FP32 → 2 KB

## Stage 12: GPU Download Embedding

cuda.memcpy_dtoh_async(...)

- Transfer embedding from GPU to CPU

- 2 KB transfer: <0.1 ms

- L2 normalize on CPU (if not already normalized)

## Stage 13: Identity Matching (CPU)

sims = known_embs @ query  # Dot product = cosine sim

best_idx = sims.argmax()

matched_id = ids[best_idx] if score >= threshold

- Hardware: CPU

- Matrix multiply: (N,512) @ (512,) = (N,) similarities

- Find best match via argmax

- Compare against dynamic/fixed threshold

- Processing time: <0.5 ms (vectorized NumPy)

## Stage 14: Async API Queue (CPU)

api_handler.send_async(name, sim)

- Add request to thread-safe queue

- Background worker thread sends HTTP POST

- Non-blocking: returns in <1ms

- Video processing never waits for API response

## Stage 15: Display (CPU)

cv2.rectangle(frame, (x1,y1), (x2,y2), color, 2)

cv2.putText(frame, label, ...)

cv2.imshow('Face Recognition [TRT GPU]', frame)

- Draw bounding boxes and labels

- Display FPS, API stats, adaptive mode indicator

- Window refresh: ~1-2 ms

# 4. TensorRT Optimization Details

## Model Conversion Pipeline

ONNX Model (*.onnx)

↓

TensorRT Builder

↓

Optimized Engine (*.engine)

### Optimization Techniques Applied:

- Layer Fusion — Conv+BN+ReLU combined into single GPU kernel

- Kernel Auto-Tuning — Selects fastest CUDA kernel per layer

- Precision Calibration — FP32/FP16/INT8 support

- Memory Optimization — Reduces VRAM usage via tensor reuse

- Platform-Specific — Optimized for Jetson's ARM+GPU architecture

### Engine File Characteristics:

- RetinaFace Engine: ~5-8 MB (FP32), ~3-4 MB (FP16)

- MobileFaceNet Engine: ~4-6 MB (FP32), ~2-3 MB (FP16)

- Platform-specific: Must rebuild on each Jetson model

- One-time build: Reused for all subsequent runs

## FP32 vs FP16 Precision

| Precision | Speed | Accuracy | VRAM |
| --- | --- | --- | --- |
| FP32 (Default) | Baseline | 100% | 100% |
| FP16 (Faster) | 1.5-2x faster | 99.9% | 50% |
| INT8 (Fastest) | 3-4x faster | 95-98% | 25% |

Recommendation: Use FP16 for 2x speedup with negligible accuracy loss. INT8 requires calibration and may reduce recognition accuracy.

## Memory Management

### Unified Memory on Jetson

- CPU and GPU share same physical RAM (8GB on Orin Nano)

- Zero-copy possible: GPU can directly access CPU buffers

- Eliminates PCIe transfer bottleneck

- CUDA managed memory automatically migrates data

### Page-Locked (Pinned) Memory

h = cuda.pagelocked_empty(...)  # Non-pageable memory

- Prevents OS from swapping memory to disk

- Enables DMA (Direct Memory Access)

- Faster GPU transfers: 2-3x vs regular memory

## Async Execution

self.stream = cuda.Stream()

cuda.memcpy_htod_async(..., self.stream)

self.context.execute_async_v3(self.stream.handle)

cuda.memcpy_dtoh_async(..., self.stream)

self.stream.synchronize()  # Wait for completion

- GPU operations queued in CUDA stream

- CPU can prepare next frame while GPU processes current

- Overlapping computation + memory transfer

- Typical overlap saves 2-3 ms per frame

# 5. Performance Analysis

## CPU vs GPU Comparison

| Platform | Detection | Recognition | Total FPS |
| --- | --- | --- | --- |
| CPU (ONNX) | 25-35 ms | 5-8 ms | ~10 FPS |
| GPU (TensorRT) | 3-5 ms | 1-2 ms | 40-60 FPS |
| Speedup | 6-8x faster | 3-5x faster | 4-6x overall |

## Detailed Latency Breakdown (Per Frame)

| Operation | CPU (ms) | GPU (ms) | Speedup | % of Total |
| --- | --- | --- | --- | --- |
| Preprocessing | 2-3 | 2-3 | 1x | 5% |
| GPU Upload (Detection) | 0 | 0.5-1 | N/A | 2% |
| Detection Inference | 25-35 | 3-5 | 6-8x | 10% |
| GPU Download (Detection) | 0 | 0.3-0.5 | N/A | 1% |
| NMS + Extract | 3-5 | 3-5 | 1x | 10% |
| GPU Upload (Recognition) | 0 | 0.1 | N/A | <1% |
| Recognition Inference | 5-8 | 1-2 | 3-5x | 4% |
| GPU Download (Recognition) | 0 | <0.1 | N/A | <1% |
| Matching | <0.5 | <0.5 | 1x | 1% |
| Display | 1-2 | 1-2 | 1x | 4% |
| TOTAL PER FRAME | ~40-55 | ~12-20 | 3-4x | 100% |

Key Insight: GPU acceleration gives 6-8x speedup on inference (stages that matter most), resulting in 3-4x overall FPS improvement.

## Expected FPS by Resolution

| Input Size | CPU FPS | GPU FPS | GPU Advantage |
| --- | --- | --- | --- |
| 320×320 | 15-20 | 60-80 | 4-5x |
| 480×480 | 12-18 | 45-60 | 3-4x |
| 640×640 (Default) | 8-12 | 35-50 | 4-6x |
| 1024×1024 | 3-5 | 15-25 | 5-7x |

Recommendation: Use 640×640 for best balance of accuracy and speed. Lower to 480×480 if need 50+ FPS.

# 6. Setup Instructions

## Prerequisites

### 1. Install JetPack 6.2.1

# Flash Jetson with JetPack 6.2.1 (includes CUDA, TensorRT, cuDNN)

# Download from: https://developer.nvidia.com/jetpack

### 2. Verify Installation

# Check CUDA

nvcc --version  # Should show CUDA 12.6

# Check TensorRT

python3 -c "import tensorrt; print(tensorrt.__version__)"

# Should show 10.3.0 or similar

# Check GPU

nvidia-smi  # Or tegrastats on Jetson

## Python Dependencies

# Install PyCUDA

pip3 install pycuda

# Install TensorRT Python bindings (if not pre-installed)

pip3 install nvidia-tensorrt

# Install other dependencies

pip3 install numpy opencv-python requests

## Model Conversion (ONNX → TensorRT)

### Convert RetinaFace

import tensorrt as trt

logger = trt.Logger(trt.Logger.INFO)

builder = trt.Builder(logger)

network = builder.create_network(1 << int(trt.NetworkDefinitionCreationFlag.EXPLICIT_BATCH))

parser = trt.OnnxParser(network, logger)

# Parse ONNX

with open('RetinaFace.onnx', 'rb') as f:

parser.parse(f.read())

# Build config

config = builder.create_builder_config()

config.set_memory_pool_limit(trt.MemoryPoolType.WORKSPACE, 1 << 30)  # 1GB

# Enable FP16 (optional, 2x faster)

config.set_flag(trt.BuilderFlag.FP16)

# Build engine

engine = builder.build_serialized_network(network, config)

# Save

with open('retinaface_fp32.engine', 'wb') as f:

f.write(engine)

Repeat similar process for MobileFaceNet. Engine building takes 5-15 minutes per model.

## File Structure

/home/socai-jetson/

├── ai_projects/

│   └── yolo_with_ultralytics/

│       └── faces/           # PKL face database

│           ├── 1.pkl

│           ├── 2.pkl

│           └── ...

├── trt_cache/               # TensorRT engines

│   ├── retinaface_fp32.engine

│   └── mobilefacenet.engine

└── data/video/              # Test videos (optional)

└── individual.mp4

# 7. Configuration Options

## Basic Settings

VIDEO_SOURCE = 0  # 0=webcam, or path to video file

INPUT_SIZE = 640  # Detection resolution (320/480/640)

CONF_THRESH = 0.7  # Detection confidence threshold

NMS_THRESH = 0.4   # IoU threshold for NMS

## Recognition Settings

USE_ADAPTIVE_THRESHOLD = False  # Recommended: keep disabled

COSINE_THRESH = 0.46           # Fixed threshold

# If adaptive enabled:

COSINE_THRESH_BRIGHT = 0.46  # Well-lit faces

COSINE_THRESH_DARK = 0.42    # Dark faces (risky!)

BRIGHTNESS_CUTOFF = 80       # Threshold boundary

## API Settings

API_ENABLED = True

CAMERA_ID = 'CAM001'

MIN_FRAMES = 5        # Frames before API call

RESET_TIME = 600      # 10-minute cooldown

## Performance Tuning

# For maximum speed (50+ FPS):

INPUT_SIZE = 480

CONF_THRESH = 0.6  # Slightly lower

# For maximum accuracy:

INPUT_SIZE = 640

CONF_THRESH = 0.75  # Stricter

# Enable FP16 in engine build (2x faster):

config.set_flag(trt.BuilderFlag.FP16)

# 8. Running the System

## Basic Execution

python3 face_recognition_tensorrt.py

## Expected Console Output

[INFO] Loading TensorRT engines...

[TRT] retinaface_fp32.engine ready

[TRT] mobilefacenet.engine ready

[INFO] Loading face database (TRT GPU)...

[INFO] Loaded 15 identities

[INFO] Async API handler started

[INFO] Adaptive Threshold: DISABLED

[INFO] Starting GPU inference...

[API ✓] User 5 - 200

[API ✓] User 12 - 200

...

## Live Display

- Window title: 'Face Recognition [TRT GPU]'

- Top-left: FPS counter (should show 35-50 FPS)

- Line 2: API statistics (success/fail/pending)

- Line 3: ADAPTIVE MODE indicator (if enabled)

- Bounding boxes: Green=known, Red=unknown

- Labels: Name (confidence) T:threshold

## Stopping the System

# Press 'Q' key to quit gracefully

# Or Ctrl+C in terminal

## Final Report Example

---------------------------------------------------------

RUN DATE: 2026-03-06 10:30:45

PIPELINE: RETINAFACE + MOBILEFACENET [TensorRT GPU]

---------------------------------------------------------

VIDEO:

- Source     : individual.mp4

- Resolution : 1920 x 1080

PERFORMANCE:

- Time       : 120.34 sec

- Avg FPS    : 42.15

- Min FPS    : 38.20

- Max FPS    : 48.90

DETECTION:

- Total Frames  : 5072

- Frames w/Face : 4891

- Known         : 4523

- Unknown       : 368

LATENCY (TensorRT GPU):

- Detection   : avg=4.12 min=3.45 max=6.78 ms

- Recognition : avg=1.67 min=1.23 max=2.34 ms

API STATISTICS:

- Total Sent: 15

- Success   : 15 ✓

- Failed    : 0 ✗

- Pending   : 0

---------------------------------------------------------

# 9. Troubleshooting

## Issue: 'No module named tensorrt'

# Solution 1: Check JetPack includes TensorRT

dpkg -l | grep tensorrt

# Solution 2: Add to PYTHONPATH

export PYTHONPATH=/usr/lib/python3.10/dist-packages:$PYTHONPATH

# Solution 3: Install pip package

pip3 install nvidia-tensorrt

## Issue: 'PyCUDA._driver.Error: cuInit failed: no device'

# Verify GPU visible

nvidia-smi

# Check CUDA installation

nvcc --version

# Reinstall PyCUDA

pip3 uninstall pycuda

pip3 install pycuda

## Issue: Low FPS (<20 on Jetson Orin Nano)

- 1. Check power mode: sudo nvpmodel -q (should be MODE_30W)

- 2. Set max clocks: sudo jetson_clocks

- 3. Lower INPUT_SIZE to 480 or 320

- 4. Use FP16 engines (rebuild with FP16 flag)

- 5. Close background apps consuming GPU

## Issue: 'Out of memory' error

# Check GPU memory usage

tegrastats

# Solutions:

# 1. Close other GPU applications

# 2. Lower INPUT_SIZE

# 3. Reduce workspace memory in engine build:

config.set_memory_pool_limit(trt.MemoryPoolType.WORKSPACE, 512 << 20)  # 512MB

## Issue: Engine file not loading

- Engines are platform-specific — rebuild on target Jetson

- Check JetPack versions match (build vs runtime)

- Verify file path and permissions

ls -lh /home/socai-jetson/trt_cache/*.engine

# 10. Advanced Optimizations

## Dynamic Batch Size

# Allow variable batch sizes (process multiple faces together)

profile = builder.create_optimization_profile()

profile.set_shape('input', (1,3,112,112), (4,3,112,112), (8,3,112,112))

config.add_optimization_profile(profile)

## INT8 Quantization (4x faster, requires calibration)

# Calibration dataset needed

config.set_flag(trt.BuilderFlag.INT8)

config.int8_calibrator = MyCalibrator()  # Custom implementation

## Multi-Stream Processing

# Process multiple cameras in parallel

stream1 = cuda.Stream()

stream2 = cuda.Stream()

# Execute on different streams simultaneously

## CUDA Graphs (Reduce Kernel Launch Overhead)

# Capture sequence of operations

graph = cuda.Graph()

graph.begin_capture(stream)

# ... execute operations ...

graph.end_capture()

graph_exec = graph.instantiate()

## DLA (Deep Learning Accelerator) — Jetson AGX Only

# Offload inference to dedicated DLA core

config.default_device_type = trt.DeviceType.DLA

config.DLA_core = 0  # Use DLA core 0

# 11. Comparison: TensorRT vs ONNX Runtime

| Aspect | ONNX Runtime (CPU) | TensorRT (GPU) |
| --- | --- | --- |
| Execution | CPU threads | GPU CUDA cores |
| Optimization | None | Layer fusion, kernel tuning |
| Precision | FP32 only | FP32/FP16/INT8 |
| Setup | Load model, run | Build engine, load, run |
| Build Time | 0 (instant) | 5-15 minutes one-time |
| Portability | Any x86/ARM CPU | NVIDIA GPU only |
| Memory | Low (~50MB) | Medium (~200-300MB) |
| FPS (640px) | 8-12 | 35-50 |
| Latency | 40-55 ms | 12-20 ms |
| Use Case | Development, CPU-only | Production, max speed |